���������������������������������������������������������������������������

[ c�a�l ] by Vikash R. Goel <vgoel@capaccess.org>

���������������������������������������������������������������������������

ABOUT CAL:

cal is a simple program for MS-DOS which allows you to exchange programs

and data between your PC and a TI calculator using a homemade piece of

linking hardware. I wrote this program to fill in a noticable gap I saw

in programs supporting the parallel port link; the only link software of

this caliber were solely for the TI-85 or the TI-92. cal was written with

two purposes in mind: to make a better MS-DOS interface for using my TI-85

with a parallel port link (compared to link85xp's interface), and to allow

TI-82 owners to use the parallel port link. Since then, I've added support

for other calculators and homemade links. cal currently facilitates transfer

to and from any of the following calculators:

TI-82

TI-83

TI-85

TI-86 (maybe)

Using any of the following link hardware:

Parallel Port Link

"$4 serial link"

Connect-85 link cable

Texas Instruments Link85 cable.

The first two above are the most common homemade links. Personally I would

recommend building the parallel link as it is the easiest and cheapest.

Note that the Link85 cable is *not* the TI-GraphLink cable. The Link85 cable

is no longer manufactured, so it is unlikely you have one. Also note that

the Link85 cable, used with cal, can be used with any of the four models of

TI calculators.

A future version of cal is likely to support the current TI-GraphLink cable,

the one currently marketed. This will make cal support all link hardware

currently in use.

For current information on all the link hardware and more, go to the

TICALC.ORG web page at http://www.ticalc.org/ and browse.

cal isn't supposed to be a replacement for other fine programs like

Connect-85 or others, nor competition or anything like that. Just use cal.

It's good, small, and fast. =)

REQUIREMENTS:

cal isn't very picky. It should run on most hardware. As far as I know or

can tell, the minimum requirements go something like this:

8086 processor

256k RAM

25k disk space

CGA display

MS-DOS 3.3 or higher

You shouldn't have too much trouble finding a system capable of running

cal. Some notes:

- TI calculators can have trouble communicating through a homemade

link with some faster processors, mainly Pentiums above 75 megahertz.

This 'fast computer problem' happens rarely with cal, while it's

common with other programs. This is because cal artificially limits

its transfer speed. It's still faster than the TI-GraphLink, but

probably slower than Connect-85 or link85xp. Note that this is done

intentionally; maybe I should make it an option which can be toggled

on and off. Feedback?

- Windows 3.x and Windows 95 are A-Okay... cal works beautifully

in a DOS session, windowed or otherwise. However, drivers for

certain parallel port devices (like a Zip Drive) may foul it up.

- I haven't tested it with Windows NT, nor have I gotten reports

about it. I strongly doubt cal would work with any version of NT

because cal uses direct hardware access.

- Non-MS DOSes like Caldera's OpenDOS or Novell's DR-DOS should

work fine, though I have no verification of this. I haven't yet

tried cal with FreeDOS, either, though I would like to.

- I have one report that cal works under dosemu with Linux! Don't

know about any other unix operating systems, though. (I imagine

you have to be root or run dosemu setuid as root for hardware port

access; that means don't let regular users use it!).

Basically, I would appreciate reports about: WinNT, OS/2, Non-MS DOSes, and

dosemu with non-Linux unices.

FILES:

The official distribution of cal is always contained in a zip file with

the name CALxyz.zip where xyz is the version. This file can be obtained

from:

http://www.geocities.com/siliconvalley/pines/2242/

Contained in this zip file are two files: CAL.DOC and CAL.EXE. CAL.DOC

is this text file. CAL.EXE is the cal executable. It's pretty much self-

contained; it doesn't create or require any other files.

HOW TO USE CAL:

Without much trouble (hopefully). I tried to make a simple user interface

with access to all the features. When you start cal, you will see a list

of all *.8?? files on the left. The top one, in a different color, is the

selected one. Up, Down, PageUp, and PageDown allow you to scroll through

the list. Pressing A will list all the files (*.*) instead of just the

TI calculator files. Uppercase entries with square brackets ([]) around

them are directories or drives. Pressing X will sort the files by their

extension, so all files with the same extension are next to each other.

To send a file to the calculator, start the receive on the calculator and

then press Enter. This will send the selected file's contents, or, if the

selected file is a directory, it will change to that directory.

To receive a file from the calculator, press R. Once cal says it is

waiting, start the send on the calculator. cal will allow you to give any

name to the file it receives. Since they are standard TI format files, it

would be advisable to use the extensions appropriate (ie 85G for a TI-85

group file, 82P for a TI-82 program, 83I for a TI-83 image, etc.).

When sending or receiving files, cal automagically detects whether the

calculator is a TI-82, 83, 85, or 86. However, if you try to send a file

to the wrong model of caluclator, it won't work. The Rx "light" turns on

when the computer is receiving or expecting a byte from the calculator. The

Tx "light" turns on when the computer is sending a byte. All the inquiries

and informational messages should appear in the small window near the middle

of the screen.

To capture the screen from the calculator, press S. cal will ask for the

type of calculator. Press 2 for a TI-82, 3 for a TI-83, 5 for a TI-85, or

6 for a TI-86. After displaying the captured screen, cal will ask for the

name of a file to save the image to; you should make the file extension BMP

as it will be a standard BMP format file.

Any time cal asks for a file name to write data to, typing nothing will

simply abort the operation. Similarly, pressing Escape while cal is expecting

input from the calculator will abort that operation.

If your link isn't connected to LPT1, pressing Tab will switch between the

ports LPT1, LPT2, LPT3, LPT4, COM1, COM2, COM3, and COM4. The LPT ports are

parallel ports and COM ports are serial ones. cal gets the addresses from

the BIOS, so nonstandard addresses shouldn't pose problems.

Some extra benefits: Delete will delete the selected file and D will shell

to DOS. Also, pressing T will allow you to test the link hardware to make

sure that you built it correctly (but don't put too much faith in its

results if it is successful; you may still have trouble). Finally, U will

decode the currently selected file using the UUDecode algorithm; many

programs are sent via e-mail using this method.

NOTE: the TI-86 support in this version has not been tested at all. As

far as I know, it may not work at all. I NEED your feedback!

BUGS, POSSIBLE BUGS, AND BUG-LIKE FEATURES IN CAL:

� Keep in mind that this is a beta release and a lot of stuff probably

doesn't work quite right.

� I don't own a TI-82, 83, or 86, so I can't really vouch for the pertinent

features. Still, I've borrowed my friends' calculators a lot and gotten

help from lots of people so they should work all right.

� I don't have any of the serial links, so I'm working on hearsay when

I create the features which support them. I've heard from people about

the "$4 serial link," but not about the Connect-85 or Link85. Anyone?

also, I have gotten one report that cal thinks COM1 is COM2 and COM2 is

COM3. Anyone else, or was that an isolated incident?

� I don't know the format of BMP files. By dissecting a few made with MS

Paint from Windows 95, I think I figured them out enough. I know the BMP

files created by cal work fine with MS Paint and Paintbrush, but can't

vouch for others.

� I'm not exactly positive about the file formats and whether or not they're

fully compatible with TI software, but I'm reasonably sure. In particular,

the TI-83 and TI-86 formats used are simply modified TI-82 and TI-85 ones.

They should be okay; right now, the only ones I'm a little worried about

are the TI-83 backups (TI-86 backups don't work at all).

� Sometimes cal ends with a "Null Pointer Assignment" error, but this

doesn't seem to interfere with anything.

� Question - Does LPT4 really exist? I can't find anyone with four parallel

ports, and MSD only lists up to three. If LPT4 is simply a figment of my

imagination, I'd like to know so I can remove it from the list.

� The TI-86 memory backups apparently don't work at all. If you have a

TI-86, I'd appreciate any kind of feedback...

� I probably haven't written them all here.

HOPES FOR THE FUTURE:

� The LCD capture is admittedly kludgey. I may even be able to make it

print for those with more than one printer port or using the serial

link. Another possibility would be saving in more standard formats

like JFIF/JPEG or GIF.

� I'm thinking of adding a UUEncode feature. Anyone feel the need?

� Maybe an edit-lock feature?

� Does anybody know how to write a ASCII85P-type program? It would be nice

to have a feature that allows you to convert 82P/83P/85P files to ASCII

and vice versa.

� A number of people have either requested a port to Linux or offered to

port it themselves. Currently, I'm not planning to give my source code

to anyone (I'd be embarrased by its quality :). Porting it would be

pretty much impossible; it uses a lot of direct hardware access, Turbo

C-specific extensions, and DOS calls, a perfect recipe for nonportability.

Creating a Linux version may be a possibility; I may try when I have

enough time. Meanwhile, try using cal with dosemu.

� I've received a number of requests for TI-92 support. Maybe one fine

day... :)

� If the display format were redesigned, it might be possible to include

support for VFAT (aka Win95 long filenames). Does anyone use long file-

names for TI calculator files? If not, would it be helpful anyhow?

� I'd like to make slightly better documentation.

ME:

I am Vikash R. Goel. You can reach me at vgoel@capaccess.org. If you have

questions about the links, you should contact the designer of the link in

question. See the addresses at the top of this document. However, if you

have trouble with either cal or a hardware link, I'd probably be glad to

try to help out.

Since the creation of cal, I've begun to get a great deal of e-mail, more

than I can handle. Sometimes I can't reply to each individual message, but

I do try to read them all. I'm wondering if I should find some way to make

a cal discussion group/mailing list kind of thing...

REGISTRATION:

This software is free. If it breaks you can keep all the pieces. However,

I disclaim all liability for what it might do to your computer, your

calculator, your social life, or yourself. If you have a problem, I would

(probably) be happy to try to help you out, but I am by no means obligated

to do so. Hey, I'm giving it away!

If you use it regularly, I'd appreciate if you would drop me a line and

tell me how it's doing. If you have suggestions, comments, or criticisms,

I'd love to hear them.

Everyone who e-mails me about cal gets placed on my "list of cal users"

and is informed when I create a new version. If you would rather not be

on this list, or want a different e-mail address on it, just e-mail me and

tell me so.

CAL HOME PAGE:

You can always get the newest public release of cal at the following url:

http://www.geocities.com/SiliconValley/Pines/2242/

You'll also find some advice about the links and stuff there.

HISTORY:

Version 0.7�

- first public release

Version 0.8�

- It turns out the BMP saving didn't work at all. Thanks to nobody for

telling me! Fixed now, at least for the TI-85. TI-82 owners, let me

know if something isn't working; I don't own a TI-82.

- BMP is no longer in inverse video.

- The bug which prevented the use of LPT1s other than 378h is fixed.

- Added support for ports LPT1 through LPT4.

Version 0.83�

- Temporary semi-public alpha

- Limited TI-83 support

- Added support for serial links

Version 0.9�

- Transition into beta status

- TI-83 support is apparently all here, except for memory backups

- Added link testing function

- Added UUDecode feature

Version 0.91�

- Attempted TI-86 support

- Added TI-83 backup support, TI-83 support now complete

- Added file list sorting, and option to sort by file extension instead of

filename

- Added directory/drive navigation

- Spruced up the display a bit -- see if you can find the new easter

egg! (it's not hard)

- Added option to display all files instead of just *.8??

- Improved UUDecoding handling, should work on all standard UUEncoded

files

- Improved LCD capture handling

- Changed LCD capture to use CGA graphics instead of VGA graphics (why

should a 2-color image require a 256-color display?!)

- Fixed bug that caused calculator to slow down after failed transmissions

- cal now 'remembers' the port setting from the last time it was run (a

lot of people, such as those using COM4, requested this feature)

- I think I've finally licked the variable-skipping problem! Now, if you're

transferring a group file to your calculator, and one already exists,

you should be able to skip that single item without aborting the entire

transfer.

- A feature I should have added a long time ago: trying to go up past the

top of the file list takes you to the bottom, and the other way around.

Also, Home and End respectively take you to the top and bottom of the

list.